SECTION 04 22 00

FLEXIBLE FLASHING TYPE 304 STAINLESS STEEL

PART 1 - GENERAL

1.01	SUMMARY

A.	Laminated stainless steel fabric flashing, non-asphaltic.

EDIT RELATED SECTIONS TO INCLUDE ONLY SECTIONS IN PROJECT MANUAL.

B.	Related sections:

1.	04 05 23	Masonry Accessories.

2.	04 21 13	Brick Masonry

3.	04 22 00	Concrete Unit Masonry

4.	04 22 23	Architectural Concrete Unit Masonry

5.	04 42 00	Exterior Stone Cladding.

6.	04 72 00	Cast Stone Masonry.

7.	05 40 00	Cold Formed Metal Framing.

8.	06 10 00	Rough Carpentry.

9.	07 11 10	Dampproofing.

10.	07 60 00	Flashing and Sheet Metal.

11.	07 65 00	Flexible Flashing.

1.02	REFERENCES

A.	Standards of the following as referenced:

1.	ASTM

2.	Brick Industry Association (BIA)

3. 	Recycled content & Recyclability

B.	Industry standards:

1.	BIA Technical Notes on Brick Construction No. 7, Water Penetration Resistance- Design and Detailing, August 2005.

2.	BIA Technical Notes on Brick Construction No. 28B, Brick Veneer/Steel Stud Walls, August 2005.

1.03	DEFINITIONS

A.	Terms:

1.	Cavity wall flashing:  Same as flexible flashing.

2.	Foundation sill flashing:  Same as flexible flashing.

3.	Flexible flashing:  Water-proof material typically used in cavity wall construction to contain and assist in the proper water drainage that may penetrate wall system veneer.  Other materials may be required to constitute the system.

4.	Head and sill flashing:  Same as flexible flashing.

5.	Through-wall flashing:

a.	Generally considered the same as flexible flashing.

b.	Rare definition referred to full width cap flashing under copings or wall caps.

1.04	SUBMITTALS

A.	Product data:  Indicate material type, composition, thickness, and installation procedures.

B.	Samples:  3" by 5" flashing material.

C.	Product Quality & Environmental submittals:

1.	Certificates:

a.	Indicate materials supplied or installed are asbestos free.

b.	Indicate recycled content:  60% total recycled material; based on 60% Post Industrial Recycled Content.

2.    Performance Attributes

a. 	Tensile strength, 100,000 psi minimum average

b. 	Puncture Resistance, 2,500 pounds average

c.    When tested as manufactured, product resists growth of mold pursuant to test method ASTM D3273.

d.	Fire Rating:  flame spread and smoke generation

1.	Rated Class A, ASTM E84

f.	Certify the use of domestic manufactured stainless steel for flashing.

g. 	Certify products contain no silica or asbestos.

1.05	QUALITY ASSURANCE

A.	Qualifications:

1.	Manufacturer:  Provide flashing materials by single manufacturer with not less than twenty-five years of experience in manufacturing flexible flashing products.

2. 	Flashing materials must be able to withstand 300º F temperature without changing the long-term performance of the flashing.

SPECIFIER’S NOTE:  All manufacturers of air barrier, insulation and flashing products should provide you MUTUAL letters of compatibility for these products in combination with each other.

1.06	WARRANTY

A.	Special warranty:

1.	Manufacturer:  Warrant flexible flashing material for life of the wall.

2.	Begin warranty at Date of Substantial Completion.

PART 2 - PRODUCTS

2.01	MANUFACTURED UNITS

- A.	Product standard of quality:

  - York Manufacturing, Inc.; Multi-Flash SS

  - Illinois Products, Inc.; IPCO Stainless Steel Fabric Flashing

  - Prosoco, Inc.; R-Guard SS ThruWall

  - STS Coatings, Inc.; Wall Guardian Stainless Steel TWF

  - TK Products, Inc.; TK TWF

  - Other products that meet the criteria in section 1.04 to 1.06.

2.	Characteristics:

a.	Type:  Stainless steel core with polymer fabric laminated to the bottom stainless steel face with non-asphalt adhesive.  The top face (exposed side) must not be covered with a polymer fabric.

b.    Stainless steel: type 304, ASTM A240.  Domestically sourced per

DFARS 252.225-7008 and/or DFARS 252.225-7009.

c.	Fabric: polymer fabric; laminated back face (non-exposed side) of stainless-steel core.

d.	Size:  Manufacturer's standard width rolls.

  - ACCESSORIES:

    - Mastic/sealant: Product standard of quality is York Manufacturing, Inc.; UniverSeal US-100.

      - Characteristics:

        - Type: One part 100% solids, solvent-free formulated silyl-terminated polyether (STPE), ASTM C920-11, Type S, Grade NS, Class 25.

    - Outside corner and inside corner material; manufacturer’s standard available units using:

      - Multi-Flash SS

      - Preformed stainless steel: 26-gauge stainless steel.

    - End dam:  Product may be folded in line with the flashing material or utilize preformed end dams by manufacturer using:

      - Preformed stainless steel:  26-gauge stainless steel

    - Splice material: Product standard of quality is York 304 SA by York. Manufacturing, Inc. standard self-adhered metal material: material matching system material or use Multi-Flash SS 6” lap piece and polyether sealant as a splice.

    - Termination bar: Product standard of quality is York T-96 termination bar.  Manufacturer's standard 1" composite material bar or a 1” 26-gauge stainless steel termination bar with sealant lip.

    - Weep vent protection:

1.Product standard of quality is York’s Weep-Armor. Geotextile drainage fabric at least 12” in height.

2. Weep-Net A free draining mesh material that will not degrade in the cavity wall and is made of polymer strands. Dovetailed design, 10” high and the dovetailed slots are 7” deep

    - Drip Edge: Stainless-steel with 30-degree 3/8” bent outer edge, hemmed. 3” by 8’

    - Weep Vents: Product standard of quality is York’s Stainless-steel weep vents. Stainless-steel mesh vent in 2.5 x 3.5 (Standard) or 3.5 x 3.5 (Jumbo).

    - Repair and other materials/accessories: Manufacturer’s standard.

    - Fasteners: Domestic manufactured fastener types and sizes recommended by flashing manufacturer for intended use.

PART 3 - EXECUTION

3.01	INSTALLATION

A.	General:

1.	Install where indicated, specified, or required in accord with flashing manufacturer's written instructions and as follows.

2.	Extend flashing 6" minimum beyond opening.  Fold flashing ends at end of openings or horizontal flashing terminations to form end dam or use pre-manufactured units made of 26-gauge stainless steel.

3.	Flashing width:  Width required starting flush with outside face of exterior wythe, extending through cavity, rising height required to extend above lintel steel at least 2".

4.	Splice end joints by overlapping them 6” and seal with a compatible sealant or metal splice tape.

5.	Masonry back up:

a.	Surface apply after dampproofing installation specified in Dampproofing Section in accord with manufacturer’s installation instructions.

b.   Fasten the top of the flashings to the back up wall with a non-corrosive termination bar and seal the top edge with a compatible sealant or use a termination clamp, which is embedded in the block back up wall.

6.	Concrete back up:

a.	Surface apply after dampproofing installation specified in Dampproofing Section in accord with manufacturer’s installation instructions.

- Fasten the top of the flashings to the back up wall with a non-corrosive termination bar and seal the top edge with a compatible sealant.

7.	Stud back up with sheathing:

a.	Fasten the top of the flashings to the back up wall with a non-corrosive termination bar and seal the top edge with a compatible sealant..

8.	Leave ready for certified compatible building felt or air barrier installation lapping flashing top installed in another Section.

9.	Lay flashing in continuous bead of sealant on masonry supporting steel.

10.	Fold ends of flashing at end of opening to form dam; seal with polyether sealant or use purchased manufacturers preformed end dams.

11.	Inside and outside corners:  Make in industry accepted manner using corner and splice material or purchase manufactured corners from manufacturer.

13.    Cover flashing within a few days of installation to protect it from damage from the different trades, the environment and falling debris. If flashing is left unprotected and it is punctured, torn, or has loose scrim you should contact the manufacturer for repair instructions.

3.02	SCHEDULES

A.	Locations:

1.	Exterior door heads.

2.	Window heads and sills.

3.	Storefront heads.

4.	Horizontal control joints.

5.	Changes in veneer materials, vertically.

6.	Other wall openings.

7.	Other locations indicated.

END OF SECTION 04 22 00